| Brick Inspection Checklist |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1. General Information |  |  |  |  |  |
| Project Name:___________________________ |  |  |  |  |  |
| Location: ______________________________ |  |  |  |  |  |
| Date of Inspection:_______________________ |  |  |  |  |  |
| Supplier Name: _________________________ |  |  |  |  |  |
| Batch/Lot No.: ____________________ |  |  |  |  |  |
| Inspected By: ________________________ Designation: ___________________________ |  |  |  |  |  |
| 2 | Visual Inspection (First Screening) | Yes (✓) | No (✗) | Remarks |
| 2.1 | Bricks are uniform in colour (deep red / copper tone preferred) |  |  |  |
| 2.2 | No visible cracks |  |  |  |
| 2.3 | Edges are sharp and corners intact |  |  |  |
| 2.4 | Free from warping/distortion |  |  |  |
| 2.5 | No excessive efflorescence (white salts) |  |  |  |
| 2.6 | Free from organic impurities / stones / lime nodules |  |  |  |
| 3 | Dimensional Check | Yes (✓) | No (✗) | Remarks |
| 3.1 | Bricks conform to specified size * |  |  |  |
| 3.2 | Variation within permissible limits (±3 mm typical) |  |  |  |
| 3.3 | Check 20 bricks stacked: total length within tolerance |  |  |  |
|  | *e.g., 190X90X90mm or as par standard |  |  |  |
| 4 | Soundness Test | Yes (✓) | No (✗) | Remarks |
| 4.1 | Strike two bricks together: Clear ringing sound |  |  |  |
| 4.2 | No dull sound (indicates internal cracks or porosity) |  |  |  |
| 5 | Hardness Test | Yes (✓) | No (✗) | Remarks |
| 5.1 | Scratch test using nail/knife: No visible impression |  |  |  |
| 5.2 | No dull sound (indicates internal cracks or porosity) |  |  |  |
| 6 | Drop Test (Impact Resistance) | Yes (✓) | No (✗) | Remarks |
| 6.1 | Drop from ~1 meter height |  |  |  |
| 6.2 | Brick should not break into more than 2–3 pieces |  |  |  |
| Comments: |  |  |  |  |
| Contractor: _____________________________ Signature _________________________ |  |  |  |  |  |
| Site Engineer: ____________________________ Signature _________________________ |  |  |  |  |  |
| QA/QC Engineer: __________________________ Signature _________________________ |  |  |  |  |  |
| Approved by: ______________________________Signature _________________________ |  |  |  |  |  |